| - Jeep Delivery Checklist |  |
| --- | --- |
| Stuff to bring for your inspection: Creeper, Flashlight, CD or phone, USB phone cord, small plug 115v light |  |
| - Verify the following properly comes with your vehicle: - Two sets of keys - Jeep tool kit for the right model - Lug nut key, if applicable - Freedom top storage bag - Floor mats – cloth (std) or rubber depending on your options - Owner’s manual – infotainment - Jeep hitch receiver rubber insert for tow group - Small baggie with tubing and splices for tow group - Black fuel filler door on Rubicon - 33inch BFG KM02 on Rubicon - Rock rails on Rubicon - _____________________________________________________ - _____________________________________________________ - _____________________________________________________ - _____________________________________________________ - _____________________________________________________ - _____________________________________________________ - _____________________________________________________ | - Verify the following are of proper quality: - Body welds – there should be nice even heating no splatter - Inspect undercarriage for any major rust - Inspect for any leaks, oil, transmission, exhaust, etc. - Inspect paint for scratches - Check oil drain plug and oil filter for proper tightening - Inspect tires and ensure minimal balancing weights were needed - Shake tires and ensure minimal play in the wheels - Inspect control arm bolts for proper torque - Verify tire pressure at manufacturer recommended 37psi - Inspect all seams for consistency - Verify all fluid levels are within acceptable range - Inspect wiring for any cuts or splices - Verify interior is clean and no soiling has occurred from the factory - Verify carpet is properly fastened and no signs of lifting - Carefully inspect the paint job for imperfections – indicate to dealer - Mileage should be under 25 miles - _____________________________________________________ - _____________________________________________________ |
| - Verify the following works as intended: - Heat and AC works including fans, etc. - Freedom Top properly removes and ensure it is properly fastened prior to leaving - Take vehicle for a quick test drive and ensure gas, brakes, clutch, etc - Test functionality of all options you ordered - Inspect all lights for functionality: interior and exterior including high beams and reverse - Windshield wipers - No warning lights are showing once started - Crank the engine and listen for any weird noises - Test the horn - Lower and Raise the windows - Steering wheel lowers and raises and telescopes as designed - Seats recline and easily move forward and backward - Safety belts - _____________________________________________________ | - Allow the salesperson to provide a full demonstration of all functionality - Install shorty antenna – Ha you are already mod’n your Jeep! - Install Infotainment Protector – Second mod while at dealership! - Fill-up gas tank when you take the vehicle for a test drive - Inquire if all recalls have been performed – such as steering prgm update - Inspect that no dealer badges were put on your vehicle - Insure you are given the vehicle registration and all paperwork - Inquire as to when the title will be provided to you, if not financing - Signed up for Jeep Wave program - Get a schedule of all maintenance required for the vehicle over its lifetime - Verify you are in 2Hi and drive home and enjoy your badass vehicle! - Remember you are in the break-in period for the engine: first 60 under 50, smooth acceleration, after 60 miles up to 55mph are desirable. While cruising (high gear) brief full-throttle acceleration for a good break-in - ________________________________________________________ |